Networks and Communication

Digital Assignment-2

Name -Diksha Singhania 						Reg no.-17BCE0032

Software-GNS3

Topic: Multicast Tunnelling

About Gns3?

Graphical Network Simulator-3 (shortened to GNS3) is network software emulator first released in 2008. It allows the combination of virtual and real devices, used to simulate complex networks. It uses Dynamips emulation software to simulate Cisco IOS.

GNS3 is used by many large companies including Exxon, Walmart, AT&T and NASA, and is also popular for preparation of network professional certification exams.

What is multicast tunnelling

In many network scenarios you want to configure your network to use GRE tunnels to send Protocol Independent Multicast (PIM) and multicast traffic between routers. Typically, this occurs when the multicast source and receiver are separated by an IP cloud which is not configured for IP multicast routing. In such network scenarios, configuring a tunnel across an IP cloud with PIM enabled transports multicast packets toward the receiver. This document describes the configuration.

Installation:

- Browse to https://gns3.com and click the Free Download link

- If you have not already registered on the GNS3 site, you will be prompted to create an account.

- Click the Download button to download the GNS3-all-in-one package

- double-click the GNS3-2.1.11-all-in-one-regular.exe

- click next until extraction finishes and installation is finished.

Components used

## Cisco 3640(ios image:- c3640-jk9s-mz.124-16.bin)

Diagram(topology):

Aims of the above:

- All IP addresses have been configured.

- Configuring OSPF on all routers to achieve full connectivity.

- We have configured dense-mode Multicast on all routers EXCEPT router Charlie.

- Configured router Echo to join the following multicast group:224.1.1.1

- A ping has been started from router Alpha to the multicast group address 224.1.1.1 to generate traffic.

- Making sure router Echo can receive the multicast stream without configuring multicast on router Charlie.

Code:

Router1(Alpha)

- Alpha#ena

- Alpha#conf t

- Alpha(config)#int f0/0

- Alpha(config-if)#no shut

- Alpha(config-if)#ip address 192.168.12.1 255.255.255.0

- Alpha(config-if)#do wr

- Alpha(config-if)#router ospf 1

- Alpha(config-router)#network 0.0.0.0 255.255.255.255 area 0

- Alpha(config-router)#^Z

- Alpha#conf t

- Alpha(config)#ip multicast-routing

- Alpha(config)#int f0/0

- Alpha(config-if)#ip pim dense-mode

- Alpha(config-if)#int f1/0

- Alpha(config-if)#ip pim dense-mode

- Alpha(config-if)#^Z

- Alpha#ping 224.1.1.1 repeat 5

- Alpha#ping 224.1.1.1 repeat 999

Router2(Bravo)

- Bravo#ena

- Bravo#conf t

- Bravo(config)#int f0/0

- Bravo(config-if)#no shut

- Bravo(config-if)#ip address 192.168.12.2 255.255.255.0

- Bravo(config-if)#int f1/0

- Bravo(config-if)#no shut

- Bravo(config-if)#ip address 192.168.23.2 255.255.255.0

- Bravo(config-if)#int f1/0

- Bravo(config-if)#no shut

- Bravo(config-if)#do wr

- Bravo(config-if)#ena

- Bravo(config)#router ospf 1

- Bravo(config-router)#network 0.0.0.0 255.255.255.255 area 0

- Bravo(config-router)#^Z

- Bravo#conf t

- Bravo(config)#ip multicast-routing

- Bravo(config)#int f0/0

- Bravo(config-if)#ip pim dense-mode

- Bravo(config-if)#int f1/0

- Bravo(config-if)#ip pim dense-mode

- Bravo(config-if)#^Z

- Bravo#show ip int brief

- Bravo#conf t

- Bravo(config)#interface tunnel 0

- Bravo#conf t

- Bravo(config-if)#tunnel source fastEthernet 1/0

- Bravo(config-if)#tunnel destination 192.168.34.4

- Bravo(config-if)#ip pim dense-mode

- Bravo(config-if)#router ospf 1

- Bravo(config-router)#passive-interface tunnel 0

- Bravo(config-router)#^Z

- Bravo#show ip int brief

- Bravo#conf t

- Bravo(config)#interface tunnel0

- Bravo(config-if)#^Z

- Bravo#ping 192.168.24.4

- Bravo#

Router3(Charlie)

- Charlie#ena

- Charlie#conf t

- Charlie(config)#int f0/0

- Charlie(config-if)#no shut

- Charlie(config-if)#ip address 192.168.23.3 255.255.255.0

- Charlie(config-if)#int f1/0

- Charlie(config-if)#no shut

- Charlie(config-if)#ip address 192.168.34.3 255.255.255.0

- Charlie(config-if)#do wr

- Charlie(config-if)#router ospf 1

- Charlie(config-router)#network 0.0.0.0 255.255.255.255 area 0

- Charlie(config-router)#^Z

- Charlie#conf t

- Enter configuration commands, one per line.  End with CNTL/Z.

- Charlie(config)#ip multicast-routing

- Charlie(config)#int f0/0

- Charlie(config-if)#ip pim dense-mode

- Charlie(config-if)#int f1/0

- Charlie(config-if)#ip pim dense-mode

- Charlie(config-if)#int f0/0

- Charlie(config-if)#no ip pim dense-mode

- Charlie(config-if)#no ip pim dense-mode

- Charlie(config-if)#int f1/0

- Charlie(config-if)#no ip pim dense-mode

- Charlie(config-if)#^Z

- Charlie#

Router4(Delta)

- Delta#ena

- Delta#conf t

- Enter configuration commands, one per line.  End with CNTL/Z.

- Delta(config)#int f0/0

- Delta(config-if)#no shut

- Delta(config-if)#ip address 192.168.34.4 255.255.255.0

- Delta(config-if)#int f1/0

- Delta(config-if)#no shut

- Delta(config-if)#ip address 192.168.45.4 255.255.255.0

- Delta(config-if)#do wr

- Delta(config-if)#router ospf 1

- Delta(config-router)#network 0.0.0.0 255.255.255.255 area 0

- Delta(config-router)#

- Delta(config-router)#

- Delta(config-router)#^Z

- Delta#

- Delta#conf t

- Delta(config)#ip multicast-routing

- Delta(config)#int f0/0

- Delta(config-if)#ip pim dense-mode

- Delta(config-if)#int f1/0

- Delta(config-if)#ip pim dense-mode

- Delta(config-if)#interface tunnel 0

- Delta(config-if)#tunnel source fastEthernet 0/0

- Delta(config-if)#tunnel destination 192.168.23.2

- Delta(config-if)#ip pim dense-mode

- Delta(config-if)#^Z

- Delta#conf t

- Delta(config)#interface tunnel 0

- Delta(config-if)#ip address 192.168.24.2 255.255.255.0

- Delta(config-if)#router ospf 1

- Delta(config-router)#passive-interface tunnel 0

- Delta(config-router)#

- Delta(config-router)#^Z

- Delta#conf t

- Delta#ping 192.168.24.2

- Delta#conf t

- Delta(config)#interface tunnel 0

- Delta(config-if)#ip address 192.168.24.4 255.255.255.0

- Delta(config-if)#

- Delta(config-if)#^Z

- Delta#ping 192.168.24.2

- Delta#debug ip mpacket

- *Mar  1 01:06:33.247: IP(0): s=192.168.12.1 (Tunnel0) d=224.1.1.1 id=542, ttl=252, prot=1, len=100(100), not RPF interface

- Delta#no de all

- All possible debugging has been turned off

- Delta#

- *Mar  1 01:06:37.243: IP(0): s=192.168.12.1 (Tunnel0) d=224.1.1.1 id=544, ttl=252, prot=1, len=100(100), not RPF interface

- Delta#show ip route

- Delta#conf t

- Delta(config)#ip mroute 0.0.0.0 0.0.0.0 tunnel 0

- Delta(config)#^Z

- Delta#debug ip mp

- Delta#no de all

- Delta#

Router5(Echo)

- Echo#ena

- Echo#conf t

- Echo(config)#int f0/0

- Echo(config-if)#no shut

- Echo(config-if)#

- Echo(config-if)#ip address 192.168.45.5 255.255.255.0

- Echo(config-if)#do wr

- Echo(config-if)#router ospf 1

- Echo(config-router)#network 0.0.0.0 255.255.255.255 area 0

- Echo(config-router)#

- Echo(config-router)#

- Echo(config-router)#^Z

- Echo#conf t

- Echo(config)#ip multicast-routing

- Echo(config-if)#ip pim dense-mode

- Echo(config-if)#

- Echo(config-if)#int f0/0

- Echo(config-if)#ip igmp join-group 224.1.1.1

- Echo(config-if)#

Screenshots:

Alpha

Output:

Bravo

Output:

Charlie

Delta

Code

Output:

Echo

Conclusion:

The Router1(Alpha) was able to receive the message from router5(echo) using the multicast tunnelling.